How to Perform a Passlock Relearn when changing the PCM or Ignition Switch Lock Cylinder:

Please read everything before beginning.

- Turn ON the ignition, with the engine OFF.

- Attempt to start the engine, then release the key to ON (vehicle will not start/engine will not crank).

- Observe the SECURITY telltale, after approximately 10 minutes the telltale will turn OFF.

- Turn OFF the ignition, and wait 5 seconds.

- Repeat steps 1 through 4 two more times for a total of 3 cycles/30 minutes (the vehicle is now ready to relearn the Passlock™ Sensor Data Code and/or passwords on the next ignition switch transition from OFF to CRANK).

Important: The vehicle learns the Passlock™ Sensor Data Code and/or password on the next ignition switch transition from OFF to CRANK. You must turn the ignition OFF before attempting to start the vehicle.

- Start the engine (the vehicle has now learned the Passlock™ Sensor Data Code and/or password).

- With a scan tool, clear any DTCs if needed (history DTCs will self clear after 100 ignition cycles).

The Above information is sourced from the GM Shop Manual.

Note: Depending on the vehicle, the SECURITY telltale can be the security light on the instrument cluster or a flashing/cycling SECURITY message on the Message Center of the instrument cluster.

Tips:

- Before beginning (before putting the key in the ignition), put the emergency brake on to disable daytime running lights (this step helps save battery power).  Make sure all doors trunk and hood are closed.  Leave the driver window down and reach in through the window to operate the ignition switch (avoids opening the doors).

- Security relearn procedure will not work with the valet key!

- Do NOT remove the key until the relearn is complete.  When the steps state to switch the ignition to the off position, just switch to off, do not remove the key from the ignition.

- Once completed and the car starts, take a moment to scan and clear codes in the system (the system may have codes remaining from the process to relearn the PCM).